XID8300 IP Address Setup

For LAN communication it is recommended  to use a static IP address. Please configure the DHCP server accordingly for the printer's MAC address (which can be retrieved in the Status Monitor tool) to prevent IP conflicts.

Start installing EDIsecure Connect with the printer turned off and unplugged.  Next install the Status Monitor (JVC Status Monitor).

Both the Status Monitor tool and EDIsecure Connect need to be installed before proceeding.

- Connect the USB cable from the computer to the XID8300, and power on the printer. This will install the corresponding USB drivers automatically.

- In the case you get an error, check if ink and retransfer films are present in the machine.  If not, insert the consumables and reset the printer, then wait for preheating.

- Launch the Status Monitor as Administrator and go to the Printer Select Tab.

- Make sure to select “Network” for default interface.

- Choose “Use the following IP Address”, and then input your IP Address, Subnet Mask, and/or Default Gateway.

- Select Update, and then close out.

- Power off the printer, disconnect the USB cable, and insert the Ethernet cable.

- Power the printer on, and wait for ready.

- Open EDIsecure Connect Administration Center.  If this is the first installation then it will ask you to install the printer.  Proceed with the printer wizard to install the necessary machines.

- Print a test card through [Devices and Printers] / [Printers and Faxes].